Standard Header (COMX <-> PC)

| Address | Bytes | Address Description | Value | Value Description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0h | 1 | Command type | 0 | None |  |  |
|  |  |  | 1 | Save |  |  |
|  |  |  | 2 | Load |  |  |
|  |  |  | 3 | Get first CAT content |  |  |
|  |  |  | 4 | Get next CAT content |  |  |
|  |  |  | 5 | CD - Change directory |  |  |
|  |  |  | 6 | MKDIR |  |  |
|  |  |  | 7 | RMDIR |  |  |
|  |  |  | 8 | DEL |  |  |
|  |  |  | 9 | Fetch time & date |  |  |
|  |  |  | 12h, 14h, 16h, 1Ah | Used in image header |  |  |
|  |  |  | 18h | COMP, Compare files |  |  |
|  |  |  | 19h | URL |  |  |
|  |  |  | 42h | DLOAD |  |  |
|  |  |  | 81h | Save and overwrite existing file |  |  |
|  |  |  | 82h | Load continue |  |  |
|  |  |  | FFh | Cancel command |  |  |
| 1h | 18 | Program or directory name |  | Name in ASCII characters |  |  |
| 13h | 1 | Result | 0 | File found |  |  |
|  |  |  | ADh, AEh, AFh | Used in image header |  |  |
|  |  |  | F3h | URL not found |  |  |
|  |  |  | F4h | No more data found |  |  |
|  |  |  | F5h | Server error |  |  |
|  |  |  | F6h | Network error |  |  |
|  |  |  | F7h | File open |  |  |
|  |  |  | F8h | File error |  |  |
|  |  |  | F9h | File size > 8K |  |  |
|  |  |  | FAh | File size > 64K |  |  |
|  |  |  | FBh | File size < 8K |  |  |
|  |  |  | FCh | File size < 64K |  |  |
|  |  |  | FDh | File type error |  |  |
|  |  |  | FEh | Address error |  |  |
|  |  |  | FFh | File NOT found |  |  |
| 14h | 2 | Actual file length (excl. header) |  |  |  |  |
| 16h | 1 | Program type | 1 | .comx machine code |  |  |
|  |  |  | 2 | .comx ‘old’ regular save/load (for backward compatibility reasons only) |  |  |
|  |  |  | 3 | .comx F&M basic save/load |  |  |
|  |  |  | 4 | .comx ‘old’ and incorrect regular data load (for backward compatibility reasons only) |  |  |
|  |  |  | 5 | .comx regular data save/load |  |  |
|  |  |  | 6 | .comx regular save/load |  |  |
|  |  |  | 7h to 9h | reserved for .comx format |  |  |
|  |  |  | 10h | .bin |  |  |
|  |  |  | 11h | .txt (memory dump) |  |  |
|  |  |  | 12h | .img |  |  |
|  |  |  | 13h | .ram |  |  |
|  |  |  | 14h | .bin, .txt, img, .ram, .comx |  |  |
|  |  |  | 20h | .comx.bak |  |  |
|  |  |  | 21h | NVSAVE / nvram.bin |  |  |
|  |  |  | 22h | SYSSAVE / system.bin |  |  |
|  |  |  | 23h | NVSYNC |  |  |
|  |  |  | 24h | URL |  |  |
|  |  |  | 25h | URL INPUT |  |  |
|  |  |  | 30h | .bin.bak |  |  |
|  |  |  | 31h | .txt.bak |  |  |
|  |  |  | 32h | .img.bak |  |  |
|  |  |  | 33h | .ram.bak |  |  |
|  |  |  | 34h | .bin.bak, .txt.bak, img.bak, .ram.bak, .comx.bak |  |  |
|  |  |  | b7 = 1 | BLOAD/BSAVE active |  |  |
| 17h | 2 | Start address |  | Basic | Machine code | Data |
|  |  |  |  | DEFUS | Start | String |
| 19h | 2 | End address |  | EOP | End-1 | Array |
| 1Bh | 2 | Exec address |  | String/Array/EOD | Exec | EOD |
| 1Ch | 18 | Filename 2 |  | Name in ASCII characters (used for USB COMP) |  |  |
| 2Fh | 1 | Category |  | Not used |  |  |
| 30h | 1 | Case type | 1 | Standard COMX characters (upper case ‘default’) |  |  |
|  |  |  | 2 | Lower case ‘default’ standard COMX characters on ‘SHIFT’ |  |  |
|  |  |  | 3 | Upper case ‘default’ and lower case characters on ‘SHIFT’ |  |  |
|  |  |  | 4 | Lower case ‘default’ and upper case characters on ‘SHIFT’ |  |  |
| 31h | 1 | High byte start load |  |  |  |  |
| 32h | 1 | BLOAD/BSAVE type | 30h | One RAM Bank x |  |  |
|  |  |  | 31h | RAM Banks 8 to 15 |  |  |
|  |  |  | 32h | RAM Banks 8 to 15 Continue |  |  |
| 33h | 2 | BLOAD/BSAVE length | 2 |  |  |  |
| 35h | 3 | FW build number |  | FW build (3 ascii values) |  |  |

CD Header (PC -> COMX)

| Address | Bytes | Address Description | Value | Value Description |
| --- | --- | --- | --- | --- |
| 0h | 1 | Result | FFh | Directory not found |
| 1h | 126 | Full path |  | Path name in ASCII characters |
| Last byte | 1 | End code | 0 |  |

URL,”url” Header (COMX -> PC)

| Address | Bytes | Address Description | Value | Value Description |
| --- | --- | --- | --- | --- |
| 0h | 1 | Command type | 19h | USB URL |
| 1h | 126 | URL |  | URL name in ASCII characters |
| Last byte | 1 | End code | 0 |  |

URL (x) and URLLINK (x) Header (COMX -> PC)

| Address | Bytes | Address Description | Value | Value Description |
| --- | --- | --- | --- | --- |
| 0h | 1 | Command type | 19h | USB URL |
| 1h | 1 | Command | < 32 | 0 = Normal USB URL / LINK 1 = PAGE BACK 2 = GO TO HOME 3 = SET HOME 4 = GO TO BOOKMARK 5 = SET BOOKMARK 6 = HELP |
| 2 | 2 | Screen number | 0-65535 | USB URL: Data screen number |
| 2 | 1 | Bookmark | 0-9 | Bookmark number |
| 2 | 1 | Link | 0-255 | USB URLLINK: New link number, link number 0 = default page |

CAT Header (PC -> COMX)

| Address | Bytes | Address Description | Value | Value Description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0h-1Bh |  |  |  | Entry 1 + x Every header contains 4 directory entries and is send until no more entries are available (entry number = 0) |  |  |
| 0h | 1 | Entry number | 0 | No more entries |  |  |
|  |  |  | 1 – FEh | Directory entry number of specified file |  |  |
|  |  |  | FFh | Directory name |  |  |
| 1h | 18 | Program or directory name |  | Name in ASCII characters |  |  |
| 13h | 1 | Result | 0 | File found |  |  |
|  |  |  | ADh | Format error SS / SD |  |  |
|  |  |  | AEh | Format error SS / DD |  |  |
|  |  |  | AFh | Format error DS / SD |  |  |
|  |  |  | F5h - FEh | Used in standard header |  |  |
|  |  |  | FFh | File NOT found |  |  |
| 14h | 1 | Size | 0 - FFh | File size if > 255 KB; otherwise 0 |  |  |
| 16h | 1 | Program type | 1 | .comx machine code |  |  |
|  |  |  | 2 | .comx ‘old’ regular save/load (for backward compatibility reasons only) |  |  |
|  |  |  | 3 | .comx F&M basic save/load |  |  |
|  |  |  | 4 | .comx ‘old’ and incorrect regular data load (for backward compatibility reasons only) |  |  |
|  |  |  | 5 | .comx regular data save/load |  |  |
|  |  |  | 6 | .comx regular save/load |  |  |
|  |  |  | 7h to 9h | reserved for .comx format |  |  |
|  |  |  | 10h | .bin |  |  |
|  |  |  | 11h | .txt (memory dump) |  |  |
|  |  |  | 12h | .img |  |  |
|  |  |  | 13h | .ram |  |  |
|  |  |  | 20h | .bak |  |  |
| 17h | 2 | Start address |  | Basic | Machine code | Data |
|  |  |  |  | DEFUS | Start | String |
| 19h | 2 | End address |  | EOP | End-1 | Array |
| 1Bh | 2 | Exec address |  | String/Array/EOD | Exec | EOD |
| 20h-3Bh |  |  |  | Entry 2 + x |  |  |
| 40h-5Bh |  |  |  | Entry 3 + x |  |  |
| 60h-7Bh |  |  |  | Entry 4 + x |  |  |

Image file Header (COMX <-> PC)

| Address | Bytes | Address Description | Value | Value Description |
| --- | --- | --- | --- | --- |
| 0h | 1 | Command type | 0 – 9 | Used in standard header |
|  |  |  | 12h | Get track image file |
|  |  |  | 14h | Save track image file |
|  |  |  | 16h | Test image file |
|  |  |  | 18h | Used in standard header |
|  |  |  | 1Ah | Get track image file continue |
|  |  |  | 42h | Used in standard header |
|  |  |  | 81h – 82h | Used in standard header |
|  |  |  | 94h | Save track image and overwrite existing file |
|  |  |  | FFh | Cancel command |
| 1h | 18 | Program or directory name |  | Name in ASCII characters |
| 13h | 1 | Result | 0 | File found |
|  |  |  | ADh | Format error SS / SD |
|  |  |  | AEh | Format error SS / DD |
|  |  |  | AFh | Format error DS / SD |
|  |  |  | F5h - FEh | Used in standard header |
|  |  |  | FFh | File NOT found |
| 14h | 1 | Density | 0 | Single |
|  |  |  | 1 | Double |
| 15h | 1 | Sides | 0 | Single |
|  |  |  | 1 | Double |
| 16h | 1 | Track type | 0 | First track |
|  |  |  | 1 | Second until last -1 |
|  |  |  | 2 | Last track |
| 17h | 1 | Drive & side | 14H | Drive 1 & side 1 |
|  |  |  | 34H | Drive 1 & side 2 |
|  |  |  | 18H | Drive 2 & side 1 |
|  |  |  | 38H | Drive 2 & side 2 |
| 18h | 1 | Track number | 0 to 34 |  |
| 19h | 1 | Side | 0 or 1 |  |
| 30h | 1 | Case type | 1 | Standard COMX characters (upper case ‘default’) |
|  |  |  | 2 | Lower case ‘default’ standard COMX characters on ‘SHIFT’ |
|  |  |  | 3 | Upper case ‘default’ and lower case characters on ‘SHIFT’ |
|  |  |  | 4 | Lower case ‘default’ and upper case characters on ‘SHIFT’ |